# ATOMiK — Product & Technical Overview

ATOMiK  ·  State-Aware Compute Architecture  ·  Pre-Seed  ·  May 2026

DATA ROOM — TECHNICAL SUMMARY

# Architecture Overview

ATOMiK is a state-aware compute primitive implemented as hardware IP (FPGA-validated) with a path to ASIC. The core equation: state = reference_state XOR accumulated_delta

Operations:

| LOAD(addr, initial_state) | Write reference state to address slot |
| --- | --- |
| ACCUM(delta) | XOR a delta into the current accumulator |
| READ() | Reconstruct current state: reference XOR accumulated deltas |
| SWAP(addr) | Commit clean epoch boundary |

# Hardware Implementation

| Platform | Zynq XC7Z020-2CLG484I (HamGeek RK-ZYNQ7020-F) |
| --- | --- |
| Soft CPU | NaxRiscv RV64GC (production) / VexRiscv SMP rv32ima (baseline validation) |
| ATOMiK core | Single-bank, 256×64-bit state table, LiteX Migen CSR |
| CSR base | 0xf0020000 (NaxRiscv SoC) |
| Linux | Ubuntu 24.04 / Buildroot 6.9, userspace access via /dev/mem |
| Bitstream | hamgeek_rk7020f.bit (LiteX synthesis, Vivado 2025.2) |
| FPGA utilization | ~63% LUT for full NaxRiscv RV64 + ATOMiK + PSUart0Bridge |

# Validated Proof

| 16/16 algebraic property tests | HARDWARE_VALIDATED — PASS on XC7Z020 through Linux userspace |
| --- | --- |
| ATOMiK Desk v0.39-K UI | HARDWARE_VALIDATED — framebuffer-native prototype on live hardware |
| AX7020 board run matrix | LIVE_MEASURED — 4-way comparison with raw artifacts |
| Formal proofs (Lean4) | SOFTWARE_VALIDATED — algebraic properties formally proved |
| SD boot artifacts | BUILD_ARTIFACT — BOOT.bin and bitstream exist; final validation in progress |

# MMIO Interface

Accessible from Linux userspace via /dev/mem at CSR base 0xf0020000. Ordering requirement: after CSR writes, issue fence iorw,iorw + dummy STATUS read before reading STATE_LO/HI to flush Wishbone pipeline.

# ASIC Path

The architecture is ASIC-portable. XOR is 1-2 gates per bit in standard cell. The 256×64-bit state table is 16KB — manageable in any modern node.

Pre-seed allocates $300K for ASIC feasibility (mentor-reviewed go/no-go). Tape-out is NOT in this round.

# Product Roadmap (High-Level)

| Now | Zynq FPGA prototype validated. Desktop UI. Algebraic proof. Workload harness ready. |
| --- | --- |
| Q3 2026 | P0 workloads measured on hardware. First paid evaluation. SD boot stable. |
| Q4 2026 | IP diligence packet. Design partner agreement(s). ASIC feasibility study. |
| 2027 | IP licensing or strategic partnership. ASIC feasibility outcome determines path. |

# Source Code & Artifacts

Private GitHub repository. Access granted under NDA.

hardware/rtl/ — ATOMiK core RTL (atomik_core_v2.v, delta_acc, state_rec)

hardware/zynq/ — Zynq SoC, FSBL, NaxRiscv integration

software/atomik_sdk/ — Python SDK, 353 tests, formal proof integration

math/proofs/ — Lean4 formal proofs of algebraic properties

results/ — Raw measurement artifacts with evidence labels